TASK 7 Dataset and Test-Scenario Plan Drowsiness, Distraction and Phone Use, version 2

| Project | Real-Time Driver Monitoring System (RTDMS) |
| --- | --- |
| Scope | How the face model and the phone, drink and food detector (YOLO26n) are tested |
| Prepared by | Youssef Medhat |
| Date | October 2026 (version 2, with the test results so far) |
| Programme | B.Sc. Graduation Project, Computer Engineering, AASTMT |

Contents

1. Introduction	3

2. What We Must Prove	3

3. Datasets for Testing	3

4. Test Scenarios	4

4.1 Drowsiness	5

4.2 Distraction and head turns	5

4.3 Phone use, drinking and eating	6

4.4 Normal driving and look-alikes (false alarms)	6

5. Recording Our Own Data	7

5.1 Setup and conditions	7

5.2 Samples needed	7

5.3 How ground truth is recorded	7

5.4 Instructions for volunteers	8

5.5 Safety rules	8

5.6 Data-collection checklist	9

6. How We Measure	9

7. Results So Far	10

7.1 Face model	10

7.2 Phone and drink detector	11

8. Privacy and Ethics	12

9. Task Checklist	13

10. References	13

# 1. Introduction

This plan explains how we test that the system catches three unsafe behaviours, and what the tests have shown so far.

Table 1: The three behaviours

| Behaviour | What it means | Detected by |
| --- | --- | --- |
| Drowsiness | Eyes closed too long, slow blinks, heavy eyelids | Face model (MediaPipe [1] + EAR [2]) |
| Distraction | Head or eyes turned away from the road | Face model (head pose + gaze) |
| Phone use, drinking, eating | Phone at the ear or in the hand, cup or food at the mouth | Object detector (YOLO26n) [3] + short rules |

# 2. What We Must Prove

The targets come from the project objectives [4]:

Table 2: Targets

| Requirement | Target | How we check it |
| --- | --- | --- |
| Eye-closure accuracy | At least 85–90% | F1 on labelled frames |
| Drowsiness alert | DROWSY within 1.5 s of closed eyes | Scenario D3 |
| Looking away | Alert after 2 s | Scenarios X1–X3 |
| Phone use | At least 85% of phone actions caught | Scenarios P1–P4, public test sets |
| Phone alert time | Under 1 second | Time from phone appearing to alert |
| False alarms | At most 1 per 5 minutes of normal driving | Scenarios N1–N5 |
| Whole-system delay | Under 500 ms | Raspberry Pi, all models running |

# 3. Datasets for Testing

We use public data (ready now, repeatable) and our own recordings (our camera and car). New since version 1: SynDD, 3MDAD and DAD.

Table 3: Test datasets and status

| Behaviour | Dataset | Size | What it gives us | Status |
| --- | --- | --- | --- | --- |
| Drowsiness | CEW [5] | 2,423 face photos | Open vs closed eyes | Done |
| Drowsiness | MRL Eye [6] | 84,898 IR eye images | Infrared eyes, like our night camera | Done |
| Drowsiness | UTA-RLDD [7] | 60 people, real video | Real drowsiness over time (PERCLOS [8]) | Planned |
| Distraction | DG-Unicamp [9] | Gaze-zone videos | Where the driver looks | Planned |
| Phone, drink | SynDD [10], test drivers | 10 drivers, 20 videos | IR, mirror view, like ours | Done |
| Phone, drink | SynDD, 5-fold check | 59 drivers in 5 groups | How much results change between drivers | Done |
| Phone, drink | 3MDAD [11] | 50 drivers, 650 clips | Unseen drivers and camera | Done |
| Phone, drink | DAD [12] | 3 drivers, 66 min normal | Unseen real infrared camera | Done |
| Phone, drink | State Farm [16] | 22,424 images | Passenger-side view: wrong angle | Dropped |
| All | Our own recordings | 10–20 volunteers | Our camera, car and lighting | Planned |

# 4. Test Scenarios

Each scenario is a short action the volunteer performs on a screen prompt. "Time × repeats" is how long one try lasts and how many tries each volunteer does.

## 4.1 Drowsiness

Table 4: Drowsiness scenarios

| ID | Scenario | What the volunteer does | Time × repeats | Pass rule |
| --- | --- | --- | --- | --- |
| D1 | Normal blinking | Looks ahead, blinks normally | 60 s × 1 | No drowsy alert |
| D2 | Slow blink | Closes eyes about 1 s | 1 s × 10 | Warning only |
| D3 | Microsleep | Closes eyes for 3 s | 3 s × 5 | DROWSY within 1.5 s |
| D4 | Heavy eyelids | Half-closed, slow eyes | 60 s × 1 | Warning when PERCLOS ≥ 15% |
| D5 | Glasses | Repeats D1–D3 with glasses | as D1–D3 | Same as without |
| D6 | Night (IR) | Repeats D1–D3 in the dark | as D1–D3 | Same as daytime |
| D7 | Sunglasses | Wears dark sunglasses | 30 s × 1 | Says "eyes not visible" |

## 4.2 Distraction and head turns

Table 5: Distraction scenarios

| ID | Scenario | What the volunteer does | Time × repeats | Pass rule |
| --- | --- | --- | --- | --- |
| X1 | Look left / right | Turns head to the side window | 3 s × 5 each | Alert after 2 s |
| X2 | Look down | Looks down at the lap | 3 s × 5 | Alert after 2 s |
| X3 | Eyes only | Head still, eyes look down | 3 s × 5 | Alert after 2 s |
| X4 | Mirror check | Quick glance at a mirror | < 1 s × 10 | No alert |
| X5 | Turned away | Turns fully away from the camera | 3 s × 3 | "Cannot see", no false drowsy |
| X6 | Face covered | Covers face with a hand | 3 s × 3 | No false alarm |

## 4.3 Phone use, drinking and eating

Table 6: Phone, drinking and eating scenarios

| ID | Scenario | What the volunteer does | Time × repeats | Pass rule |
| --- | --- | --- | --- | --- |
| P1 | Call, right ear | Holds a phone to the right ear | 15 s × 3 | Alert within 1 s |
| P2 | Call, left ear | Holds a phone to the left (far) ear | 15 s × 3 | Alert within 1 s |
| P3 | Texting at the wheel | Types with the phone at chest height | 20 s × 3 | Alert within 1 s |
| P4 | Texting in the lap | Types with the phone low, looking down | 20 s × 3 | Warning at 2 s, alert at 3 s |
| P5 | Phone in a holder | Taps a phone in a dashboard holder | 15 s × 2 | No phone alert |
| E1 | Drinking | Drinks from a bottle, then a cup | 15 s × 3 | Alert within 1 s |
| E2 | Eating | Eats a snack | 30 s × 1 | Recorded; not yet detectable |

## 4.4 Normal driving and look-alikes (false alarms)

Table 7: Normal driving scenarios

| ID | Scenario | What the volunteer does | Time × repeats | Pass rule |
| --- | --- | --- | --- | --- |
| N1 | Normal driving | Sits as if driving, checks mirrors | 5 min × 1 | ≤ 1 false alarm |
| N2 | Talking, smiling, singing | Talks and laughs naturally | 2 min × 1 | No alert |
| N3 | Hands on the wheel | Turns the wheel, moves hands on it | 60 s × 1 | No phone alert |
| N4 | Hand at the face | Scratches face, hand on chin, adjusts glasses | 30 s × 1 | No alert |
| N5 | Passenger | A passenger talks and uses a phone | 60 s × 1 | No alert for the driver |

Why N3 and N4 are new On the unseen DAD camera the detector mistook the steering wheel in the hand for a phone, and on 3MDAD a hand at the mouth (smoking) caused false alerts. These two scenarios check that this does not happen with our camera.

# 5. Recording Our Own Data

## 5.1 Setup and conditions

Table 8: Recording setup

| Item | Plan |
| --- | --- |
| Volunteers | 10–20 drivers (at least 10): mixed gender, age and skin tone, with and without glasses |
| Camera | Project camera (Pi 5 + 160° infrared camera) at the centre mirror |
| Video | 1296 × 972 full view, 30 frames per second |
| Conditions | Pass A in daylight; pass B at night (IR light on) or in a garage, with sunglasses or a hat |
| Props | Phone + phone holder, bottle, cup or can, a snack, wipes |
| Car | Parked, engine off, handbrake on; the volunteer only mimes driving |
| Time per person | About 25 minutes (two 6-minute guided passes + extra scenarios + setup) |

## 5.2 Samples needed

Table 9: How many examples 10 volunteers give

| Behaviour | Per volunteer | Total (10 volunteers) |
| --- | --- | --- |
| Eyes closed 3 s (D3) | 5 × 2 lighting | 100 |
| Looking away (X1–X3) | 20 | 200 |
| Phone calls (P1, P2) | 6 + 4 in the guided passes | 100 |
| Texting (P3, P4) | 6 + 2 in the guided passes | 80 |
| Drinking (E1) | 3 + 4 in the guided passes | 70 |
| Normal driving and look-alikes | about 15 minutes | about 2.5 hours |

2.5 hours of normal driving lets us measure false alarms far better than the 35 minutes per group we had on SynDD.

## 5.3 How ground truth is recorded

- The recording tool shows one prompt at a time ("Phone to your right ear now") and saves the time of every prompt. These times are the labels, so no manual labelling is needed.

- The 0.5 s around each change is skipped (the volunteer is still reacting).

- A second team member checks 10% of the clips by eye.

- The guided pass follows a fixed 6-minute script (below).

Table 10: The 6-minute guided pass

| Time (s) | Prompt | Label |
| --- | --- | --- |
| 0–20 | Sit normally, eyes on the road | Normal (sets the driver's baseline) |
| 20–35 / 45–60 | Phone call, right ear / left ear | Phone at ear |
| 70–90 | Texting, phone held low | Phone in hand |
| 100–115 | Phone in the holder, quick tap | Look-alike |
| 125–165 | Drink from a bottle, then a cup or can | Drinking |
| 175–205 | Eat a snack | Eating |
| 215–250 | Talk out loud, then sing | Look-alike |
| 260–320 | Hand on head, scratch face, mirror glances | Look-alike |
| 320–340 | Nod down-up ×3, turn left-right ×3 | Head-pose check |

## 5.4 Instructions for volunteers

- Sit as you normally drive, hands on the wheel.

- Do only what the screen asks, then go back to looking ahead.

- Act naturally: hold the phone and cup the way you usually do.

- Say "stop" at any time; we stop at once and can delete your video.

- Do not start the engine or move the car.

## 5.5 Safety rules

- Never on a real road. Car parked, engine off, handbrake on (or a driving simulator).

- A team member stays with the volunteer the whole time.

- Eyes-closed and looking-away tests last at most 3 seconds.

- IR light is the camera's built-in low-power light only.

- Volunteers can stop at any time with no reason needed.

## 5.6 Data-collection checklist

Table 11: Checklist for every session

| When | Check | Done |
| --- | --- | --- |
| Before | Consent form signed | ☐ |
| Before | Car parked, engine off, handbrake on | ☐ |
| Before | Camera sees the head, both ears, hands, lap and wheel | ☐ |
| Before | Props ready: phone, holder, bottle, cup, snack | ☐ |
| Before | Lighting noted (day / night), glasses / sunglasses noted | ☐ |
| During | Calibration: 10 s looking ahead, 3 s eyes closed | ☐ |
| During | Pass A (day) and pass B (night or garage) recorded | ☐ |
| During | Extra scenarios D1–D7, X1–X6, N1–N5 done | ☐ |
| After | Prompt-time file saved next to each video | ☐ |
| After | Video copied to the project computer only | ☐ |
| After | 10% of clips checked by a second person | ☐ |

# 6. How We Measure

Table 12: Measures and success criteria

| Measure | Simple meaning | Success |
| --- | --- | --- |
| Recall (caught) | Of all real actions, how many raised an alert | ≥ 85% for phone calls and drowsiness |
| Precision | When it alerted, how often it was right | As high as possible |
| F1 score | One number balancing the two | ≥ 0.85 |
| False alarms | Alerts while the driver was fine | ≤ 1 per 5 minutes (12 per hour) |
| Delay | Time from the action starting to the alert | Drowsy ≤ 1.5 s, distracted ≈ 2 s, phone < 1 s |
| Coverage | How often the face could be seen at all | ≥ 95% in normal driving |

Results are reported per group (day / night, glasses, each volunteer), not only as one average. An average can hide a group the system serves badly.

# 7. Results So Far

## 7.1 Face model

Tested on the CEW test set (1,696 photos) [5] against two other approaches:

Figure 1: Face model compared with two other eye-state models (CEW test set)

Table 13: Eye-state results on CEW (laptop)

| Model | F1 | Accuracy | False alarms | Speed per frame |
| --- | --- | --- | --- | --- |
| MediaPipe + EAR (our model) | 0.945 | 94.6% | 5.1% | 5.1 ms |
| Haar cascades [14] | 0.817 | 80.7% | 26.1% | 9.5 ms |
| YuNet [15] + CNN | 0.933 | 93.5% | 5.7% | 11.8 ms |

Figure 2: Face model F1 under damaged images: above 85% in every case

## 7.2 Phone and drink detector

Tested at alert level: did the system raise the right alert during the action, and how many false alarms happened in normal driving? Settings were chosen on validation drivers; test drivers were scored once.

Table 14: SynDD test drivers (10 drivers never seen in training)

| Behaviour | Earlier detector (v2b) | Current detector (v2d) |
| --- | --- | --- |
| Phone at right ear | 85% | 80% |
| Phone at left ear | 100% | 100% |
| Texting | 41% | 41.5% (46% with warnings) |
| Drinking | 95% | 90% |
| Eating | 0% | 0% |
| False phone alarms per hour | 1.7 | 0 |

Figure 3: Share of actions caught on three test sets

Figure 4: False phone alarms per hour of normal driving

Table 15: Results on data never used in training

| Test set | Phone calls | Texting | Drinking | False phone alarms |
| --- | --- | --- | --- | --- |
| 3MDAD (50 drivers) | 80–92% | 96% right hand, 16% left (phone hidden) | 68–80% | 8% of normal clips |
| DAD front camera (real IR) | 4 of 5 | 5 of 6 | 2 of 2 | 69 per hour |
| DAD top camera (real IR) | 4 of 5 | 5 of 6 | 0 of 2 | 258 per hour |

What we learned The detector finds phones well on new drivers: 80–100% of phone calls on SynDD and 3MDAD. False alarms depend on the camera position. On the mirror view they are low (0–9 per hour); on DAD's close-up camera the steering wheel was mistaken for a phone. Eating is not detectable yet (0% everywhere), and texting with the phone below the camera is often missed; a head-down rule now adds a warning. So our own recordings are the deciding test, and their normal driving will be added to training.

# 8. Privacy and Ethics

- Every volunteer signs a consent form and can stop at any time.

- Videos stay on the project computer only; never uploaded or put on GitHub.

- Raw videos are deleted after labelling; only labels and numbers are kept.

- No volunteer's face appears in a report without written permission.

- Public datasets are used under their licences (research use only where required).

# 9. Task Checklist

Table 16: Testing tasks and status

| # | Task | Status |
| --- | --- | --- |
| 1 | Test the face model on CEW (1,696 photos) | Done |
| 2 | Compare with two other eye models | Done |
| 3 | Test the detector on 10 held-out SynDD drivers | Done |
| 4 | 5-fold check on 59 SynDD drivers | Done |
| 5 | Test on unseen data (3MDAD, DAD) | Done |
| 6 | Guided recording tool (prompts + saved times) | Done |
| 7 | Consent form | Planned |
| 8 | Record 10–20 volunteers, day and night | Planned |
| 9 | Run all scenarios on the recordings | Planned |
| 10 | Measure speed on the Raspberry Pi with all models | Planned |
| 11 | Write the final results report | Planned |

# 10. References

[1]  Google. MediaPipe Face Mesh V2 and BlazeFace model cards; MediaPipe Face Landmarker documentation. ai.google.dev/edge/mediapipe

[2]  T. Soukupová, J. Čech. "Real-Time Eye Blink Detection using Facial Landmarks." CVWW, 2016.

[3]  G. Jocher, J. Qiu. Ultralytics YOLO26. docs.ultralytics.com, 2025.

[4]  RTDMS project team. Real-Time Driver Monitoring System, Project 1 Final Report. AASTMT, 2026.

[5]  F. Song, X. Tan, X. Liu, S. Chen. "Eyes Closeness Detection from Still Images with Multi-scale Histograms of Principal Oriented Gradients." Pattern Recognition, 47(9):2825–2838, 2014. (CEW)

[6]  R. Fusek. "Pupil Localization Using Geodesic Distance." ISVC 2018, Springer, 2018. (MRL Eye Dataset)

[7]  R. Ghoddoosian, M. Galib, V. Athitsos. "A Realistic Dataset and Baseline Temporal Model for Early Drowsiness Detection." IEEE CVPR Workshops, 2019. (UTA-RLDD)

[8]  D. F. Dinges, R. Grace. "PERCLOS: A Valid Psychophysiological Measure of Alertness as Assessed by Psychomotor Vigilance." US FHWA Tech Brief FHWA-MCRT-98-006, 1998.

[9]  DG-Unicamp Driver Gaze Dataset, University of Campinas (as selected in the Project 1 report).

[10]  M. S. Rahman et al. "Synthetic Distracted Driving (SynDD2) Dataset for Analyzing Distracted Behaviors and Various Gaze Zones of a Driver." arXiv:2204.08096, 2022. (AI City Challenge 2024 Track 3)

[11]  I. Jegham, A. Ben Khalifa, I. Alouani, M. A. Mahjoub. "A Novel Public Dataset for Multimodal Multiview and Multispectral Driver Distraction Analysis: 3MDAD." Signal Processing: Image Communication, 88:115960, 2020.

[12]  O. Köpüklü, J. Zheng, H. Xu, G. Rigoll. "Driver Anomaly Detection: A Dataset and Contrastive Learning Approach." IEEE WACV, 2021. (DAD)

[13]  CVAT.ai Corporation. Computer Vision Annotation Tool (CVAT). github.com/cvat-ai/cvat

[14]  P. Viola, M. Jones. "Rapid Object Detection using a Boosted Cascade of Simple Features." IEEE CVPR, 2001.

[15]  W. Wu, H. Peng, S. Yu. "YuNet: A Tiny Millisecond-level Face Detector." Machine Intelligence Research, 20(5):656–665, 2023.

[16]  State Farm. State Farm Distracted Driver Detection, Kaggle, 2016. (Used in version 1 only.)